ESP32 - TFT LCD

| TFT Pin | ESP32-S3 Pin | Purpose |  |
| --- | --- | --- | --- |
| VCC | 5V | Power |  |
| GND | GND | Ground |  |
| CS | GPIO14 | Chip Select |  |
| RESET | GPIO13 | Reset |  |
| DC | GPIO12 | Data/Command |  |
| MOSI | GPIO11 | SPI MOSI |  |
| SCK | GPIO10 | SPI Clock |  |
| LED | 3.3 V | Backlight |  |
| MISO | GPIO9 | SPI MISO |  |

### Keypad 4×3

C3 → GPIO5

C2 → GPIO6

C1 → GPIO7

R4 → GPIO15

R3 → GPIO16

R2 → GPIO17

R1 → GPIO18

Ultrasonic

VCC → 3.3 V

GND → GND

Trig → GPIO 3

Echo → GPIO 46

Buzzer

BUZZER “+” → 3V3

BUZZER “–” → NPN collector (e.g., 2N2222, S8050, BC547)

NPN emitter → GND

ESP32 GPIO4 → 10 kΩ → NPN base

100 kΩ from base to GND (optional pull-down)

1 kΩ resistor in parallel with the buzzer (“+” to “–”) as the sheet suggests for charging/discharging the piezo.

Common GND between ESP32 and the transistor side.

Temp & Humid

| BME280 Pin | ESP32-3Pin | Function |
| --- | --- | --- |
| VIN | 3V3 | Power |
| GND | GND | Ground |
| SCK | GPIO42 | I²C Clock (SCL) |
| SDI | GPIO41 | I²C Data (SDA) |

Accelerometer

| MPU6050 | ESP32-S3 Pin |  |  |
| --- | --- | --- | --- |
| VCC | 5 V |  |  |
| GND | GND |  |  |
| SCL | GPIO 2 |  |  |
| SDA | GPIO 1 |  |  |